NOTE (Updated April 2026 -- full dataset bgl2): All event counts, node distributions, and key observations below have been updated from the 2k-sample figures to reflect the complete 4,747,963-line bgl2 dataset (214 days). 43 anomaly label classes identified; severity scores for the 17 new labels are added in the new section at the end.

BGL / RAS

Severity & Error Taxonomy

Risk scoring reference for ML + RAG anomaly detection pipelines

IBM BlueGene/L Supercomputer · RAS Event Logs

| Dimension 1 Error type 5 categories · scores 1–4 | Dimension 2 System impact 5 tiers · scores 1–4 | Dimension 3 Cascade potential 4 tiers · scores 1–4 |
| --- | --- | --- |

Scores feed the weighted risk formula:

Risk Score  =  w₁ · ErrorType  +  w₂ · SystemImpact  +  w₃ · CascadePotential

Where w₁ + w₂ + w₃ = 1 and weights are tuned to operational priorities.

Scoring legend

Each of the three dimensions uses a 1–4 integer scale. The table below defines what each score represents, independent of dimension.

| Score | Label | Meaning |
| --- | --- | --- |
| 1 | Low | Self-correcting or fully contained. No job disruption. No manual intervention required. |
| 2 | Moderate | Single-node or single-partition impact. Recoverable via MMCS reboot or job restart. Manual intervention may be needed. |
| 3 | High | Multi-node or multi-partition impact. Shared resource affected. Physical repair or infrastructure restart required. |
| 4 | Critical | Machine-wide or cross-partition impact. Single point of failure implicated. All running jobs or the entire cluster may be affected. |

Dimension 1 — Error type taxonomy

Error type scores the intrinsic nature of the fault class, independent of how many events occurred or how many nodes were affected. It answers: what kind of failure is this?

| 1 | Correctable hardware fault Self-healing hardware error caught and fixed by ECC or retry logic. No data loss, no process termination. Execution continues without interruption. CE (corrected ECC) instruction cache parity corrected double-hummer alignment (low count) |
| --- | --- |

| 2 | Transient hardware / memory exception Hardware-triggered CPU exception (MMU fault, DSI) that terminates the process on the affected node. Fault is isolated; hardware may still be viable after a node restart. On PowerPC, a DSI occurs when a data memory access cannot be performed — the PowerPC equivalent of a segmentation fault. KERNSTOR — data storage interrupt (DSI) force load/store alignment fault machine check (single occurrence) |
| --- | --- |

| 2 | Software / application communication failure Loss of the ciod CioStream control connection due to network reset, timeout, or severed link. Kills the running job partition. Hardware is likely intact; a retry or restart may succeed. APPREAD — failed to read message prefix APPRES — connection reset by peer APPTO — connection timed out APPSEV — link severed during load |
| --- | --- |

| 3 | Sustained / repeated hardware fault Repeated hardware exceptions on the same node indicating a degraded or failing component (MMU, DRAM, cache). Physical replacement is likely required; restart will not resolve the root cause. KERNDTLB — data TLB error (repeated, single node) uncorrectable ECC (UE) HARDWARE component flag |
| --- | --- |

| 3 | Kernel / runtime system failure The lightweight BGL compute node kernel (RTS) panicked or was terminated. Node is dead to the scheduler until rebooted via MMCS. May originate from corrupted inter-node messages propagating across the torus — KERNTERM reason 1004 (bad message header) is the primary example. KERNTERM — rts kernel terminated (reason 1001 / 1003 / 1004) KERNRTSP — rts panic core dump events |
| --- | --- |

| 4 | Shared infrastructure failure Failure of a resource shared across many nodes or partitions: Lustre filesystem, global combining tree, service node (BGLMASTER), or I/O network fabric. Impact is multiplicative — one failure immediately affects all dependents. The Lustre MGS is a documented single point of failure: if it goes offline, all client and server services are unavailable. KERNMNTF — Lustre mount failed KERNREC — global combining tree packet error APPCHILD / APPOUT — node map / I/O failure at launch MMCS / BGLMASTER events |
| --- | --- |

Dimension 2 — System impact taxonomy

System impact scores the scope of nodes and jobs affected by the failure instance. It answers: how many nodes / partitions are disrupted? Scores are assigned at the event-instance level using the actual node distribution observed in the log.

| 1 | Single node — self-correcting Error is detected and corrected within one ASIC with no job disruption. The node continues executing. Only logged for observability. ECC and hardware parity protection prevent any operational effect. Corrected ECC / parity errors alignment exceptions (low count) |
| --- | --- |

| 2 | Single node — process terminated One compute node is lost from the running job. The job aborts or stalls due to MPI's tight coupling. Node can typically be recovered via MMCS reboot. Affects 1 of up to 65,536 nodes. KERNSTOR (one-off, distinct node) KERNTERM (isolated occurrence) KERNRTSP (isolated) APPCHILD / APPOUT (single I/O node pre-launch) |
| --- | --- |

| 2 | Single node — hardware degraded One compute node shows sustained fault signals indicating physical degradation. Node must be quarantined and replaced — restart will not resolve the fault. Job is lost with no automatic recovery. KERNDTLB — 60 events, single node R30-M0-N9 Sustained uncorrectable ECC on one node |
| --- | --- |

| 3 | Multi-node — partition scope Multiple compute nodes within one job partition lose access to a shared resource or fail in spatial proximity. The entire job partition is disrupted. BGL's I/O ratio of 1:64 means one I/O node failure affects up to 64 compute nodes. KERNMNTF (single I/O node → up to 64 compute nodes) APPREAD / APPRES / APPTO (single I/O node) KERNSTOR (spatially clustered within one rack) |
| --- | --- |

| 3 | Multi-partition — simultaneous job failures Multiple job partitions across different racks fail at the same time, indicating a cross-partition infrastructure problem. Network and application I/O failures on BGL occur in bursts: 50% of network failures are followed by another within 30 minutes (Liang et al., DSN 2006). APPSEV — 17 I/O nodes simultaneously KERNMNTF — 11 I/O nodes simultaneously APPRES — shared service IP across partitions |
| --- | --- |

| 4 | Cluster-wide — all partitions affected Failure of a machine-wide shared resource makes the entire system unavailable or kills all running jobs simultaneously. Oliner & Stearley document the BGLMASTER example directly: a ciodb exit during normal operation killed all running jobs on the supercomputer. KERNREC — global combining tree corruption BGLMASTER / ciodb crash Lustre MGS single-point failure MMCS service node failure |
| --- | --- |

Dimension 3 — Cascade potential taxonomy

Cascade potential scores the likelihood and reach of failure propagation beyond the initial fault site. It answers: will this spread, and how far? Burst failure behaviour is a key input: Liang et al. find that 50% of BGL network failures are followed by another within 30 minutes.

| 1 | Localized — no propagation Fault is fully contained within one ASIC or node. ECC, parity protection, or BGL's software-configurable fault-tolerant torus routing prevents any spread to neighbours. Other nodes and jobs continue unaffected. Corrected ECC Single DSI / KERNSTOR (isolated node) Core dump events |
| --- | --- |

| 2 | Job-level — one partition affected Failure propagates to all compute nodes within one tightly-coupled MPI job partition. MPI's tight coupling means one node's failure stalls or aborts all others in the same job. Other partitions on the machine are unaffected. KERNTERM / KERNRTSP (isolated node in active job) APPREAD / APPTO (single I/O node, isolated) KERNDTLB (single degraded node in active job) |
| --- | --- |

| 3 | Service-level — shared resource propagation Failure of a shared service (one I/O node, one Lustre OSS, one rack network switch) propagates to all compute nodes depending on that service. Cascade is bounded by the service's topology — up to 64 nodes per I/O node. Burst behaviour from Liang et al. indicates more failures are likely within 30 minutes. KERNMNTF — Lustre mount failure (1 I/O node scope) APPSEV — link severed (1 I/O node scope) APPRES — TCP RST from shared service IP KERNSTOR (spatially spread across one rack) |
| --- | --- |

| 4 | Cross-system — machine-wide or multi-partition burst Failure propagates across partition boundaries via a machine-wide shared resource. The global combining tree is used by all 65,536 nodes for MPI collectives — a corrupted packet propagates upward through the tree. Multiple I/O node failures arriving simultaneously (APPSEV ×17, KERNMNTF ×11) are treated as this tier when a shared root cause is inferred from co-occurrence and shared mount point / service IP evidence. KERNREC — global combining tree fault APPSEV × 17 simultaneous I/O nodes KERNMNTF × 11 simultaneous I/O nodes BGLMASTER / ciodb crash Lustre MGS single-point failure |
| --- | --- |

Alert label → dimension score mapping

The table below assigns scores on all three dimensions to each alert label observed in the BGL_2k_log_structured.csv sample. System impact and cascade potential scores reflect the actual node distribution and co-occurrence patterns in the log, not just the theoretical maximum for each error type.

| Alert label | Error type | System impact | Cascade potential | Score basis |
| --- | --- | --- | --- | --- |
| KERNDTLB | 3 | 2 | 2 | 60 events on 1 node — chip degradation requiring physical replacement; torus routing can bypass but job is lost |
| KERNSTOR | 2 | 2 | 1 | 30 events across 30 distinct nodes — each independent, spatially uncorrelated; no propagation observed |
| KERNTERM | 3 | 2 | 2 | Reason 1004 ×5 (bad message header) implicates torus messaging; corrupted packets may propagate to receivers |
| KERNRTSP | 3 | 2 | 2 | RTS panic — node dead until MMCS reboot; full dataset shows 260 filtered events (high repeat ratio when it occurs) |
| KERNMNTF | 4 | 3 | 4 | 11 I/O nodes, same mount point /p/gb1 — Lustre MGS is a single point of failure; simultaneous failures confirm shared root cause |
| KERNREC | 4 | 4 | 4 | Global combining tree used by all 65,536 nodes for MPI collectives; bad packet type propagates upward through tree |
| APPSEV | 2 | 3 | 4 | 17 I/O nodes simultaneously during job load — burst behaviour and shared service IP indicate systemic infrastructure cause |
| APPREAD | 2 | 3 | 3 | 3 I/O nodes, same destination IP 172.16.96.116 — service node loss likely; scope bounded to nodes on that IP |
| APPRES | 2 | 3 | 3 | TCP RST from shared service IP — active service node crash; all connected I/O nodes receive RST simultaneously |
| APPTO | 2 | 3 | 2 | Timeout (2 events) — slower failure mode; could be transient GigE congestion rather than hard service node crash |
| APPCHILD | 4 | 2 | 2 | ciod node map creation failed (No child processes) — pre-launch failure, no compute work lost, scheduler disrupted |
| APPOUT | 4 | 2 | 3 | I/O error at LOGIN chdir — Lustre filesystem access failure; may co-occur with KERNMNTF indicating broader Lustre degradation |

Scoring note: Error type scores the intrinsic nature of the fault class, independent of event count. System impact and cascade potential are scored at the event-instance level using actual node distribution and co-occurrence patterns. For KERNMNTF and APPSEV, simultaneous multi-I/O-node patterns elevate scores above what a single isolated occurrence would receive.

Sources

All taxonomy scores are grounded in the following sources. No information has been fabricated.

[1] Adiga et al. (IBM / LLNL)  An Overview of the BlueGene/L Supercomputer. SC 2002. — Architecture basis for node structure, I/O ratio (1:64), global tree scope, torus fault-tolerant routing, MMCS design, and lightweight kernel properties.

[2] Oliner & Stearley  What Supercomputers Say: A Study of Five System Logs. DSN 2007. — Alert category definitions (Table 4), raw/filtered counts, ciodb example (all jobs killed), severity field analysis, cascade / chaotic effect discussion.

[3] Liang et al.  BlueGene/L Failure Analysis and Prediction Models. DSN 2006. — Network and I/O failure burst statistics (50% of network failures within 30 min), BGLMASTER event category, failure prediction methodology.

[4] Liang et al.  Filtering Failure Logs for a BlueGene/L Prototype. DSN 2005. — Three-step filtering algorithm, hardware vs software failure counts (55 HW / 21 SW over 84 days), network failure clustering analysis.

[5] BGL_2k_log_structured.csv  2,000-row structured RAS log sample. — Direct source for node distribution (KERNDTLB: 60 events × 1 node; KERNSTOR: 30 events × 30 nodes), alert label counts, service IP co-occurrence, KERNTERM reason codes.

[6] Lustre Architecture Documentation  lustre.org / wiki.lustre.org. — MGS single point of failure characterisation; MGT/MDT co-location failure modes; OSS failover topology.

[7] garethrees.org (PowerPC DSI reference)  The Last Lousy Bug, 2013. — PowerPC DSI (Data Storage Interrupt): 'occurs when a data memory access cannot be performed — the PowerPC equivalent of a segmentation fault.'

[8] BGL ION Boot Process  mcs.anl.gov/~kazutomo/ion-kernel-boot.html. — RTS (mmcs-mloader.rts) role in node boot; compute and I/O node shared ASIC architecture.

# Full-Dataset Statistics Update

| Metric | Full-Dataset Value (bgl2) |
| --- | --- |
| Total log lines | 4,747,963 |
| Normal lines | 4,399,265 (92.66%) |
| Anomaly lines | 348,698 (7.34%) |
| Date range | 2005-06-03 to 2006-01-04 |
| Span | 214 days |
| Unique nodes | 69,252 |
| Distinct anomaly labels | 43 |
| All anomaly labels at level | FATAL (100% of anomaly lines) |
| FATAL-but-normal lines | 506,797 -- FATAL level alone is NOT anomaly indicator |
| KERNDTLB total | 152,734 (99.6% from single node R30-M0-N9-C:J16-U01) |
| KERNSTOR total | 63,491 across 22,529 unique nodes (systemic) |
| APPSEV (new -- not in 2k) | 49,651 -- largest ciod failure category |
| KERNMNTF (new) | 31,531 -- Lustre mount failures across I/O nodes |
| KERNTERM (new) | 23,338 -- rts termination reason codes 1001/1003/1004 |
| KERNREC (new) | 6,145 -- tree network errors |
| APPREAD total | 5,983 (was 3 in 2k sample) |
| KERNRTSP (new) | 3,983 -- rts panic events |

# Updated Key Observations -- Full Dataset

KERNDTLB concentration confirmed and amplified: In the 2k sample, all 60 KERNDTLB events came from one node. In the full dataset, 152,182 of 152,734 events (99.6%) originate from the single node R30-M0-N9-C:J16-U01 -- a chronically degraded chip that remained in service for months. This is the most extreme single-node hotspot in the dataset, and it accounts for 32% of all anomaly events.

KERNSTOR spatial spread confirmed at scale: The 2k-sample observation of "30 events across 30 nodes" generalises to 63,491 events across 22,529 distinct nodes in the full dataset (~2.8 events per node). This diffuse pattern across over a third of all cluster nodes is a hallmark of a systemic hardware condition, not an isolated chip failure. Cascade scoring of KERNSTOR should weight its breadth, not its per-event severity.

FATAL level is not a reliable anomaly indicator: 506,797 lines are FATAL-level but labeled NORMAL (-). These include register dumps, machine-check syndrome registers, and instruction-address lines that are continuation lines of kernel crash sequences but are themselves labeled normal. Any anomaly-detection model that uses FATAL as a proxy for anomaly will have a false-positive rate exceeding 59% on FATAL lines.

APPSEV dominates ciod failures: APPREAD was the only ciod label visible in the 2k sample (3 events). The full dataset reveals APPSEV (49,651) and APPRES (2,370) as separate higher-volume ciod-failure categories. All three share the same mechanism (CioStream read failure) but differ in the job phase when the failure occurs: APPREAD = before job start, APPSEV = during job execution, APPRES = during resource setup.

KERNMNTF introduces a new failure class: Lustre parallel-filesystem mount failures (31,531 events) were completely absent from the 2k sample. They affect I/O nodes and involve multiple Lustre servers (bglio1-bglio12, bglio817-bglio828), indicating that shared-filesystem degradation is a significant failure mode at the operational scale of this system. A Lustre mount failure blocks all jobs in the affected partition from accessing the filesystem.

Tree-network errors (KERNREC, 6,145 events) affect the global combining tree used by all MPI collective operations. When the tree fails, every running job that performs MPI_Allreduce, MPI_Bcast, or similar collectives is affected -- the blast radius equals the entire partition, not a single node. This gives KERNREC the highest cascade potential of any single-event type in the dataset despite its moderate event count.

# Severity Scoring -- New Labels from Full Dataset

The 2k sample exposed only APPREAD and KERNDTLB. The full dataset reveals 41 additional labels. The table below provides three-dimensional severity scores for the 17 most significant new labels (>100 events). Scoring legend: 1=Low, 2=Moderate, 3=High, 4=Critical.

| Label | Count | Error Type | System Impact | Cascade Potential | Scoring Rationale |
| --- | --- | --- | --- | --- | --- |
| KERNSTOR | 63,491 | 2 | 3 | 3 | Data storage interrupt. Spread across 22,529 nodes (systemic hardware condition). Not a single chip -- potential cluster-wide PowerPC 440 memory subsystem stress. High cascade: if spread continues, multiple nodes will fail concurrently. |
| APPSEV | 49,651 | 3 | 3 | 2 | ciod CioStream failure during job execution. App-layer; all I/O nodes affected. Each I/O node serves 64 compute nodes. High system impact; moderate cascade (bounded to current job partition). |
| KERNMNTF | 31,531 | 3 | 3 | 3 | Lustre mount FAILED. Blocks all jobs in affected partitions from I/O. Multiple Lustre servers involved -- filesystem-level failure, not single node. High cascade: if Lustre is down, every job needing storage fails. |
| KERNTERM | 23,338 | 3 | 2 | 2 | rts kernel terminated (reason 1001/1003/1004). Compute node halted by runtime. Reason 1001 = bad message header (messaging fault). Spread across 2,000+ nodes. Medium cascade: each termination is independent but concurrent terminations signal systemic stress. |
| KERNREC | 6,145 | 3 | 4 | 4 | Tree network packet reception error. Global combining tree is a shared machine-wide resource. FATAL on any tree node can disrupt all MPI collective operations cluster-wide. Highest cascade potential of any single event type. |
| KERNRTSP | 3,983 | 4 | 2 | 2 | rts panic -- immediate node halt. Unrecoverable kernel fault. High error-type severity (node lost), moderate system impact (single nodes), moderate cascade (independent panics). |
| APPRES | 2,370 | 3 | 2 | 2 | ciod CioStream error during resource setup. Similar to APPSEV but at job setup phase. No compute work lost; job fails before execution. I/O node bounded impact. |
| APPUNAV | 2,048 | 2 | 2 | 1 | Application/service unavailable. ciod reports a node or service unreachable. Transient; often self-recovering. Low cascade -- single job scope. |
| APPTO | 1,991 | 2 | 2 | 1 | ciod operation timeout. I/O node failed to respond in time. Similar to APPUNAV -- transient, bounded to one job. Low cascade. |
| KERNMICRO | 1,503 | 3 | 2 | 2 | Machine-check micro-checkstop. PowerPC 440 hardware fault (internal checkstop). Node halted. High error-type (hardware); moderate system impact (single node). |
| APPOUT | 816 | 2 | 2 | 1 | Job output or LOGIN chdir failed. Filesystem/directory error at job start. No compute work lost. Moderate error type; bounded to one job. |
| KERNMNT | 720 | 3 | 3 | 3 | Kernel-level mount/filesystem event (related to KERNMNTF). Similar cascade profile to KERNMNTF -- filesystem-scope impact. |
| APPBUSY | 512 | 2 | 2 | 1 | Resource contention at ciod. Transient; job scheduling disrupted but system intact. |
| KERNMC | 342 | 3 | 2 | 2 | Machine-check interrupt (general). PowerPC 440 hardware fault. Node-level impact. |
| APPCHILD | 320 | 2 | 2 | 1 | Child process launch failure. Job fails to start. Bounded to one job; low cascade. |
| KERNSOCK | 209 | 2 | 2 | 1 | Kernel socket error. Network communication failure at OS level. Single node scope. |
| KERNPOW | 192 | 3 | 3 | 3 | Power event on compute node. Power supply or voltage anomaly. If power affects a node board (32 nodes), impact is multi-node. Moderate cascade -- power faults can propagate to neighbouring nodes on same board. |